FLIGHT DISPATCHER PROCEDURE

## Section 3: Normal procedures

### 3.1. End-to-end process flow

#### 3.1.1. Flight plan preparation & risk assessment

| Step | Task | Time | Notes |
| --- | --- | --- | --- |
| Step 1 Flight data check and assess | Check and assess all factors which have impact on the flight. | Before flight planning |  |
| Check flight schedule | Check the aircraft data and schedule time for flights |  |  |
| Check type of flights | Check type of flights (VIP, ferry, charter, normal flight or cargo flight,…) |  |  |
| Check NOTAM – refer to FDM 6.2.2.6 | Check all NOTAMs which have impact on the flight – refer to FDM 6.2.2.6 |  |  |
| Check RFFS available of airports – refer to FDM table 4.2 | Check RFFS available of departure, enroute, destination, alternate airports, and enroute airports that have impact on the flight – refer to FDM table 4.2 |  |  |
| Check WX – refer to FDM 4.2.3/ 6.2.2.7/ 6.2.4/ EDTO operation manual/ airport charts | Check WX departure, enroute, EDTO ERA, destination, alternate airports, and for the planned en-route airspace using VNA weather tools and official meteorological sources, including government aviation weather services, that may impact the flight – refer to FDM 6.2.2.7; |  |  |
| Check Zero Fuel Weight (ZFW) | Check the payload and passenger information for flights; Check DOW information for any type of aircrafts in AHM560 |  |  |
| Check aircraft performance defects – refer to FDM 6.2.2.9 | Check any MEL/CDL defect that received from MOC that have impact on the flight |  |  |
| Check take-off performance (RTOW) and landing limitations – refer to FDM 4.2.4 | Check take-off and landing performance or limitations via FlySmart for Airbus aircrafts, OPT for Boeing aircrafts and RTOW document folders for any type of aircrafts in EFB. |  |  |
| Check flight permission | Check day/ time/ type of aircraft and routes in flight permissions for any flights. |  |  |
| Check crew names and crew qualification | Check PIC/ FO names and crew qualifications for any flights |  |  |
| Taxi and APU fuel/ time | Check extra fuel/ taxi fuel update information in FON |  |  |
| Other information (cost index, fuel, other operation information) – refer to FON in EFB | Check performance factors and cost index for any aircrafts update in FON; Check extra fuel/ taxi fuel/ holding fuel update information in FON; Check other information operations update in FON. |  |  |
| Step 2 Decision | Decide after checking all information | Flight planning |  |
|  | Perform produce flight plan based on above checked information |  |  |
|  | If risks have been identified and exceeded predefined thresholds, flight dispatcher shall: + Discuss with OCC DM + Mitigate prior to release, or + Escalate for operational decision. |  |  |
| Step 3 Transmission ATC FPL and briefing package/ Records | Complete and records (if any) |  |  |
|  | Complete operation flight plan (OFP); Noted threats documented and other information in dispatch notes if necessary; Watchlist designation (if applicable); File ATC FPL and send OFP briefing package. | The upload time for OFP before ETD: + The earliest is 240m; + The lasted is 70m. |  |

#### 3.1.2. Dispatch release

##### Authorized dispatch release operators:

Only flight dispatchers with valid and qualified Dispatch Release licenses (FD certification) are permitted to perform dispatch releases and only their names shall be listed on the OFP.

##### Responsibilities:

DSP performing Dispatch Release (electronic or paper-based) shall perform the following:

+ Produce the OFP and be fully responsible for their output;

+ Be responsible for the ATC files for assigned flights;

+ Prepare, update, upload, and verify flight documentation in VNA-approved systems and ensure that all flight documents under the dispatcher’s responsibility are accurate, complete and up to date before performing the Dispatch Release.

+ Dispatch Release must not be performed later than: for domestic flights: ETD-60m; for international flights: ETD-90m.

Absolutely do not perform Dispatch Release until all factors related to flight safety, mission completion capability, etc., are ensured, or until any issues remain unclear or thoroughly resolved.

##### Dispatch Release between shifts:

Previous shift DSP:

+ Upload documents to EFB as usual.

+ Dispatch Releases are only permitted for flights scheduled to depart after the end of the 90m shift.

+ Hand over and highlight flights uploaded to EFB but not yet dispatched.

Next-shift DSP:

+ Receive shift handover and highlight flights uploaded to EFB but not yet dispatched.

+ Responsible for updating OFPs, flight documents, crew briefings, etc., for flights that have not yet departed since taking over the shift.

+ OFPs created since taking over the shift (new OFPs or updated OFPs) must include the name of the FD who performed the refresh/update.

+ Responsible for performing dispatch releases on all OFPs uploaded to EFB during the shift (new or updated OFPs).

##### Release procedure:

| Step | Task | Time | Notes |
| --- | --- | --- | --- |
| Step 1 Verify | Verify dispatch completeness |  |  |
|  | Confirm all planning elements are compliant and current |  |  |
|  | Ensure alternates, fuel, and routing remain valid at release time |  |  |
| Step 2 Prepare | Prepare dispatch release package |  |  |
|  | Prepare OFP and all documents related to the flight |  |  |
|  | Special instructions or monitoring notes to flight crew if necessary |  |  |
| Step 3 Obtain approval | Obtain supervisory approval (if required) |  |  |
|  | For the flight with heightened risk and operational complexity, flight dispatchers need to release the flight under supervisory |  |  |
| Step 4 Dispatch release | Release the flight |  |  |
|  | Release flight to flight crew via approved channels in VNA |  |  |
|  | Ensure clarity of any conditions or monitoring expectations |  |  |

#### 3.1.3. Post-release change management (Amendments)

| Step | Task | Time | Notes |
| --- | --- | --- | --- |
| Step Post-release change management | Management any amendments related to the flight after flight release | Before flight departure |  |
| Monitor any information changes which have impact on the flight | Inform VIAGS/ Station manager when reissuing the OFP version and contact the Pilot, if necessary, such as: + TOW changes according to FON; + Pilot’s name change; + New ETD/ ETA if exceed more than 30 minutes; + Change additional/ extra fuel and destination/ enroute alternate airports according to FON due to WX change (ex: new TAF published), new MEL/ CDL defect,…; + Change flight route due to NOTAM, avoid hazard weathers (typhoon, tropical storm, volcanic ash,…) or delay due to ATC separation; + New MEL/ CDL which has impact on flight operation (like APU INOP, performance penalties, change flight level, EDTO not operated, PBN not operated). |  |  |

#### 3.1.4. Flight watch/ En-route monitoring

##### Purpose:

To ensure that all weather and/or NOTAM changes that may impact on the continued and planned operations are monitored and action taken if necessary;

To ensure all ACARS received from the crew are monitored and action taken if necessary;

To ensure all SITA messages received are monitored and action taken if necessary.

##### Flight watch and en-route monitoring procedure

The primary system used for flight watch and en-route monitoring is VNA-approved system and Flightradar24;

Flight dispatchers shall monitor the progress of flights under his/her responsibility using VNA-approved systems. Flight monitoring includes tracking the progress of the flight from “Off-block” to “On-block”;

The specific position shall monitor NOTAM/ WX data, and incoming ACARS messages from crews; this position shall monitor incoming SITA messages and action those that have any impact on any VNA flights. The staff of this specific position shall evaluate the information received, identify the flights affected, design an action plan and then brief the respective other positions to take any action;

Any ACARS or SATCOM communications should be performed by the specific position;

The flight monitoring and flight watch shall cease once the flight has terminated;

The staff of specific position should brief the DM and DGM OCC regarding any issue that may impact on the operation.

Notes:

+ If weather conditions at EDTO ERA deteriorate below landing minima, or the EDTO ERA becomes unavailable for another reason, the staff of specific position shall attempt to select another suitable ERA to support the EDTO route segment and if necessary transmit a revised OFP to the aircraft via ACARS.

+ If communication between the aircraft and flight dispatcher is lost due to technical limits of communication system, the flight dispatcher shall take any necessary actions to make the data and information which need to be sent to the aircraft available at the next flying area where the communication will be restored. The most reliable method should be used by flight dispatcher is via ATC center of FIRs where the aircraft is flying over. If the communication is lost due to a permanent failure of communication system, the flight dispatcher shall notify the OCC DM and DGM and keep close contact with ATC center over which aircraft will fly, and standby for further actions as demanded by the management of OCC

| Step | Task | Time | Notes |
| --- | --- | --- | --- |
| Step 1 Flight watch and flight following | Perform flight watch and flight following | After departure |  |
| Flight watch | Perform flight watch for all flights, including: + Continuous monitoring of flight progress; + Surveillance of weather, NOTAMs, traffic flow, ATFM constraints, fuel exposure, and operational risks; + Early identification of trends that may adversely affect flight safety or operational integrity. |  |  |
| Flight following | Perform flight following, focusing on: + General situational awareness; + Detection of abnormal events or developing risks requiring intervention or escalation |  |  |
| Step 2 In-flight information support | Support to provide necessary information to flight crew |  |  |
|  | The position shall ensure that relevant operational information is provided to flight crews for flights under responsibility; |  |  |
|  | For all flights: + Updates when there are significant weather changes (en-route or airport) that may adversely affect the flight. |  |  |
|  | For EDTO flights: + Operational and weather updates transmitted 15-30 minutes prior to entering the EDTO sector. |  |  |
|  | For flights operating through Uncontrolled Airspace: + Operational updates provided prior to entry into uncontrolled airspace. |  |  |
|  | For flights operating to Uncontrolled Airports: + Operational and weather updates provided prior to commencement of approach. |  |  |
|  | For flights that are holding while awaiting diversion or requesting a change due to unforeseen circumstances (technical, medical, passenger, etc.): send updates as soon as information is received. |  |  |

#### 3.1.5. Post flight

Record pilot reports and feedback after the flight termination (if any);

If necessary, prepare reports on operational incidents that occur during the shift (divert, AQD report, etc.).

### 3.2. Duty-based dispatcher roles

#### 3.2.1. Flight dispatcher – Specific role in supervising and monitoring

##### Description of main roles:

This position takes responsibility for managing the overall effectiveness of the Vietnam Airlines Flight Dispatch section under the term of the AOC and in accordance with applicable CAAV and company rules and regulations. This position is the key interface with the direct management of assigned leader and other functions within the OCC. This position is designated as a Flight Dispatcher (FD).

##### Responsibilities:

The unit is manned 24 hours per day and 7 days per week.

- Procedural compliance & governance:

+ Responsible for following the procedures contained in this document unless circumstances dictate otherwise;

+ Exercising supervisory judgment when circumstances require deviation from standard procedures, and ensuring such deviations are properly assessed, documented, and escalated.

- Operational oversight & situational awareness

+ Maintain situation awareness regarding weather, NOTAM, MEL/ CDL data and other operational events and ensure such information is accurate and updated;

+ Monitor all (including VIP and Charter) flights under his/her area of responsibility.

- Escalation, reporting & management notification

+ Advise the DM, GM and DGM OCD of any event that will or may impact on the flight operations and provide alternative option(s) if applicable;

+ Advise the DM, GM and DGM OCD of any significant event and any event that may impact the performance of the Flight Dispatch unit.

- Coordination and information dissemination

+ Provide Maintenance Operations Control (MOC), Control Load Center (CLC), crew tracking, station managers and external unit like AIS and ATC with pertinent information that may impact on the operation from a dispatcher perspective;

+ Provide MOC, DM and DGM OCC with weather information for stations that are forecasted to have snow, icing and/or freezing precipitations;

+ Provide timely information to flight crew whenever necessary that may impact on the safety of flight from a dispatcher perspective;

+ Manage ad-hoc requests from other members of the OCC teams;

- Operational problem management

+ Responsible for identifying and analyzing any operational problems affecting safety or legality that may affect normal operations and co-ordinate and direct corrective action to ensure schedule integrity, safety, security and customer service, as may be required;

- Leadership and shift continuity

+ Ensure that all applicable members of the OCC are aware and updated with any operational factors that have occurred and may affect the safety progress or operations of a flight, including those flights that have been released;

+ Receive and preside over the shift handover and disseminate instructions to other positions during the shift; preside over handling any situations that arise during the shift; conduct the shift handover at the end of the shift.

##### Specific procedure:

| Step | Task | Time | Notes |
| --- | --- | --- | --- |
| Step 1 Shift handover | Perform shift handover | 15 minutes before the end of previous shift |  |
| Receive handover | Perform shift handover with the previous shift and other relevant positions. + Receive handover about the status of current situation from previous shift; + Receive handover regarding notes about flight permission flight schedule, tracking, and other relevant positions; + Important notes from the management of OCC. |  |  |
| Gather information | Gather important information about: + NOTAM, WX, military operations, technical status, MEL, detect, aircraft malfunction AOG; + Flights need to be noted, other outstanding issues. |  |  |
| Check systems | Check systems and work tools like VHF, ACARS, SATCOM, iPad and telephone with sufficient battery (above 20%), and other systems and tools. Check stable 4G/ Wifi connection, necessary applications and documents up to date. |  |  |
| Step 2 Overall review | Perform review operational plan | Beginning of the shift |  |
| Review operational plan | Check the number of flights per day to allocate to each dispatcher position appropriately. Check special flights (VIP, charter, test flight if any). |  |  |
| Dissemination | Disseminate handover information to dispatcher positions. Confirm that positions have understood the special instructions for the shift. |  |  |
| Step 3 Flight preparation | Prepare documents for assigned flights | During the shift |  |
| Check relevant information | This step is referred to 3.1. End-to-end process flow |  | All documents must be prepared 03 hours before departure for DOM flights, 04 hours for INT flight |
| Step 4 Dispatch release | Produce flight plan and release |  |  |
|  | Use flight planning system to produce flight plan |  |  |
|  | Perform flight release refer to 3.1. End-to-end process flow |  |  |
| Step 5 Post-release change management | Monitor any information changes that impact flight plan | Before flight departure |  |
|  | This step is referred to 3.1. End-to-end process flow |  |  |
| Step 6 Flight watch and flight following | Perform flight watch and flight following | After departure |  |
| Flight watch | This step is referred to 3.1. End-to-end process flow |  |  |
| Flight following | This step is referred to 3.1. End-to-end process flow |  |  |
| Step 7 In-flight information support | Support to provide necessary information to flight crew |  |  |
|  | This step is referred to 3.1. End-to-end process flow |  |  |
| Step 8 Detect, assess, and manage of operational situations | Perform management of operational situations | During the shift |  |
|  | Upon identification of an operational situation, this position shall: - Assess the potential impact on: + Flight safety and legality; + Network or OCC performance; - Take the lead in coordinating actions among: + Other flight dispatchers positions; + DM; + Maintenance Operations Control (MOC); + Other relevant positions; - Direct or approve corrective actions within assigned authority to mitigate risks and maintain operational continuity. |  |  |
|  | Manage ad-hoc operational requests from OCC positions and supporting units. |  |  |
| Step 9 Reporting and escalation | Perform reporting and escalation | If necessary |  |
|  | Advise the DM, GM and DGM OCD without delay of: + Any serious or potentially serious operational event; + Any situation likely to significantly impact safety, legality, or the flight schedule; |  |  |
|  | Provide management with: + Situation overview; + Operational impact assessment; + Feasible alternative options when applicable. |  |  |
| Step 10 Documentation and shift continuity | Prepare for next shift handover | End of shift |  |
|  | Ensure that significant operational developments, decisions, and actions are: + Properly recorded in the shift handover file or log. |  |  |
|  | Preside over the shift handover process by: + Receiving the handover at the beginning of the shift; + Disseminating instructions and priorities during the shift; + Conducting a structured handover at the end of the shift to ensure continuity of operations. |  |  |

##### Qualifications:

This position requires all staff must be at least 21 years of age. Minimum education requirements is University education. Good verbal and written in English. Undergo the fundamental aviation course by aviation institutes from Vietnam or international institutes or FTC, and other training courses as required by the Training Manual of VNA. Hold a Flight dispatcher license by the CAAV. Minimum of 05 years’ experience in a flight dispatcher environment.

#### 3.2.2. Flight dispatcher – Specific role in mastering NOTAM and WX

##### Description of main roles:

This position is responsible for the monitoring, assessment, and management of NOTAM and weather information relevant to Vietnam Airlines flight operations, in accordance with the terms of the AOC and applicable CAAV and company rules and regulations. The position ensures that NOTAM and weather data affecting flight safety, legality, and operational continuity are accurate, current, and appropriately disseminated within the OCC. This position is designated as a Flight Dispatcher (FD).

##### Responsibilities:

The unit is manned 24 hours per day and 7 days per week.

- Procedural compliance and governance

+ Perform all duties in accordance with the procedures and requirements set out in this manual, unless operational circumstances require otherwise.

+ Exercise professional judgment when deviations from standard procedures are necessary due to operational conditions, ensuring that such deviations are appropriately assessed, documented, and escalated in accordance with established OCC procedures.

- NOTAM and WX monitoring

+ Monitor NOTAM and WX information relevant to Vietnam Airlines flight operations to maintain continuous situational awareness of conditions affecting safety, legality, and operational continuity.

+ Maintain oversight of NOTAM and WX developments affecting departure, destination, alternate aerodromes, en-route airspace, and relevant FIRs.

- Data verification and accuracy control

+ Verify that NOTAM and WX data obtained from approved sources are accurate, current, and operationally valid.

+ Identify outdated, conflicting, or incomplete information and take appropriate action to clarify, correct, or escalate such issues.

- Operational assessment and impact evaluation

+ Assess the operational impact of NOTAM and WX conditions on flight planning, routing, alternates, fuel requirements, and in-flight operations.

+ Support other positions in identifying and mitigating operational risks arising from adverse WX or NOTAM constraints.

- Information dissemination and coordination

+ Disseminate relevant NOTAM and WX information in a timely manner to other flight dispatchers, and other OCC functions, as required.

+ Ensure that flight crews receive timely and accurate NOTAM and WX updates that may affect flight safety or operational decision-making.

- Escalation and reporting

+ Escalate significant NOTAM or WX-related issues that may impact flight safety, legality, or operational continuity to the appropriate supervisory or management level.

+ Promptly report abnormal, rapidly changing, or high-risk conditions in accordance with OCC escalation procedures.

- Shift continuity

+ Ensure that significant NOTAM and WX updates, assessments, and operational advisories are properly documented to support traceability, post-flight review, and operational analysis.

+ Communicate ongoing NOTAM and WX concerns, unresolved issues, and flights requiring continued monitoring during shift handover to ensure continuity of safe operations.

##### Specific procedure:

| Step | Task | Time | Notes |
| --- | --- | --- | --- |
| Step 1 Shift handover | Perform shift handover | 15 minutes before the end of previous shift |  |
| Receive handover | Perform shift handover with the previous shift and other relevant positions. + Receive handover about the status of current situation from previous shift; + Receive handover regarding notes about outstanding and ongoing tasks related to NOTAM and WX, flight permission, slot. |  |  |
| Check systems | Check systems and work tools like VHF, ACARS, SATCOM, iPad and telephone with sufficient battery (above 20%), and other systems and tools. Check stable 4G/ Wifi connection, necessary applications and documents up to date. |  |  |
| Check NOTAM/ WX | Check the current DSP briefing (most recent update time) and the prominent NOTAM/ WX alerts of the day. |  |  |
| Step 2 Overall review | Perform review NOTAM/ WX |  |  |
| Assess NOTAM/ WX | Determine shift work plans: assess WX, NOTAM/ flight operations, update alternates, optimize routes, and support dispatch and flight watch. |  |  |
| Dissemination | Disseminate handover information about NOTAM/ WX to dispatcher positions. Confirm that positions have understood the special instructions for the shift. |  |  |
| Step 3 Flight preparation | Prepare documents for assigned flights | During the shift |  |
| Check relevant information | This step is referred to 3.1. End-to-end process flow |  | All documents must be prepared 03 hours before departure for DOM flights, 04 hours for INT flight |
| Step 4 Dispatch release | Produce flight plan and release |  |  |
|  | Use flight planning system to produce flight plan |  |  |
|  | Perform flight release refer to 3.1. End-to-end process flow |  |  |
| Step 5 Post-release change management | Monitor any information changes that impact to flight plan | Before flight departure |  |
|  | This step is referred to 3.1. End-to-end process flow |  |  |
| Step 6 Flight watch and flight following | Perform flight watch and flight following | After departure |  |
| Flight watch | This step is referred to 3.1. End-to-end process flow |  |  |
| Flight following | This step is referred to 3.1. End-to-end process flow |  |  |
| Step 7 In-flight information support | Support to provide necessary information to flight crew |  |  |
|  | This step is referred to 3.1. End-to-end process flow |  |  |
| Step 8 WX assessment | Assess weather for VNA flight network | During the shift |  |
| Collect meteorological data periodically | Check TAF at 05:00, 11:00, 17:00, and 23:00 UTC; Compare take-off/landing standards and alternate airport conditions on the approved systems in VNA; Identify substandard or near-standard airports within the ETD/ETA timeframe. |  |  |
| Analyze actual trends | Check at least 4 of the most recent METAR reports at the airport that need attention Assess the trend: improving/ deteriorating/ fluctuating; Hourly stability level Monitor clouds/rain/wind by area (assess direction of movement and extent of impact) + Check satellite imagery for at least the last 2 hours + Identify the direction of movement of thunderstorms and the affected area. |  |  |
| Summary | Summarize and draw conclusions for the entire network + Determine the level of operational risk according to the planned operating time frame + Mark groups requiring special monitoring (close to standard/below standard/rapidly fluctuating) |  |  |
| Decision on extra fuel | Make Extra Fuel decisions based on specific airports/route + Clearly state the basis (TAF/METAR/satellite/trend) + Record in the shift log and prepare content for briefing updates + Discuss with SUP/ DM when the impact is widespread or likely to spread |  |  |
| Step 9 NOTAM assessment | Assess NOTAM for VNA flight network | During the shift |  |
| Establish the scope of the review | Scanning of NOTAMs valid for at least 7 days from the date of assignment Thorough review: arrival airport, alternate airport, FIR/related air routes. |  |  |
| Classify and assess the level of impact | Flight route/FIR: AWY closure, time restrictions, prohibited/restricted areas, military activities; Airport group: RWY/TWY, SID/STAR/APP, curfew, RFFS, ground/handling limitations; CNS/Navaid Group: radar/comm/navaid impacting ATC capacity and flight procedures; Determine the effective time and timeframe of impacts from overlapping ETD/ETA. |  |  |
| Determine impact and decide | Determine route/FL impact, alternate flight options, operating hours, ATC approval capability + Proposed remedial actions alternative route (route/APP/alternate) and notify relevant departments for agreement. Decide extra fuel regulations when affecting military operations at the airport + Apply a minimum of 20 minutes of extra fuel for the affected flight (if applicable according to internal regulations). + Clearly state the notice on OFP/Release when necessary. |  |  |
| Step 10 Airport evaluation and operation limits issue. | Evaluate and update the list of eligible alternate airports; issue daily/shift operation limits. | During the shift |  |
| Summary of assessment | Use WX and NOTAM/HĐQS results as a basis for selecting alternate airports based on time slots |  |  |
|  | Refer to news/ assessments from POD at daily update points (or equivalent based on actual conditions) |  |  |
| Selection and updating of alternate airports | Selection based on criteria and practical feasibility (operating hours, NOTAM, RFFS, handling) |  |  |
|  | Identify airports that are “not recommended/temporarily out of use” when risk is high (substandard, curfew, restricted RFFS, closed RWY/APP…) |  |  |
| Issuing operational limits | Publish daily/shift limits and notify other DSP positions for consistent application when developing OFPs/ releases/ handling situations |  |  |
| Step 11 DSP Briefing | Provide DSP briefing | During the shift |  |
| Content synthesis and selection | Synthesize WX, NOTAM/ Military action plan, operational limits, MEL/CDL status (if any) |  |  |
|  | Prioritize content directly affecting safety/ on-time performance/ operational capacity |  |  |
| Compile briefings | Compile briefings according to a unified structure + WX/ NOTAM by airport/ region; + Cloud and satellite maps; +Assess time slots where weather phenomena may affect flight operations at specific airports; + Traffic status, other special alerts |  |  |
| System update and version management | Update approval systems in VNA at scheduled times: 02:30, 05:30, 08:30, 11:30, 14:30, 17:30, 21:30, 23:30 UTC Specify update time/version |  |  |
| Perform ad-hoc updates when major changes occur | When WX/NOTAM changes affecting safety or operations: immediately update and promptly report to SUP/ DM and relevant positions. |  |  |
| Step 12 Documentation and shift continuity | Prepare for next shift handover | End of shift |  |
|  | Ensure that significant operational developments, decisions, and actions are: + Properly recorded in the shift handover file or log. |  |  |

##### Qualifications:

This position requires all staff must be at least 21 years of age. Minimum education requirements is University education. Good verbal and written in English. Undergo the fundamental aviation course by aviation institutes from Vietnam or international institutes or FTC, and other training courses as required by the Training Manual of VNA. Hold a Flight dispatcher license by the CAAV.

#### 3.2.3. Flight dispatcher – Specific role in controlling automatically calculated rules

##### Description of main roles:

This position is responsible for the monitoring, control, and management of automated operational rules within VNA-approval system, in accordance with the terms of the AOC and applicable CAAV and company rules and regulations. The position ensures that such rules are correctly configured, updated, and applied for each duty period to reflect current NOTAM, weather information, and operational directives, thereby supporting flight safety, regulatory compliance, and operational continuity. This position is designated as a Flight Dispatcher (FD).

##### Responsibilities:

The unit is manned 24 hours per day and 7 days per week.

- Procedural compliance and professional judgment

+ Perform all duties in accordance with approved procedures governing automated operational rules and system configuration.

+ Exercise professional judgment when automated rules require adjustment due to operational conditions, ensuring that all changes are assessed, justified, documented, and escalated in accordance with OCC governance procedures.

- Monitoring and control of automated operational rules

+ Monitor automated operational rules within VNA-approved systems to ensure correct functionality and continued operational validity.

+ Maintain oversight of rule execution to confirm that automated actions are applied as intended for each duty period.

(iii) Configuration and update of rules

+ Configure, activate, deactivate, or adjust automated operational rules as required to reflect: current NOTAM; WX conditions; operational directives and OCC instructions.

+ Ensure that rule configurations are correctly aligned with the applicable operational context of the duty period.

- Validation and consistency assurance

+ Verify that automated rules are applied consistently across flights with similar operational characteristics.

+ Ensure that newly created or modified rules do not conflict with, override, or duplicate existing rules or actions.

- Support to Automated and Manual Flight Planning

+ Support automated flight planning processes by ensuring that rule-based logic produces operationally valid outputs.

+ Coordinate with other positions when automated rule outcomes require review, refinement, or manual intervention.

- Anomaly detection and corrective action

+ Identify errors, misconfigurations, or unintended consequences arising from automated rule application.

+ Implement corrective actions at the individual flight level or rule-configuration level, as appropriate, to maintain safety and compliance.\

- Reporting and escalation

+ Escalate significant rule-related issues, system conflicts, or risks to flight safety, legality, or operational continuity to the appropriate supervisory or management level.

+ Provide timely notification when automated rules may impact flight operations or require operational awareness.

- Shift continuity

+ Ensure that all rule changes, adjustments, and exceptions are properly recorded to support traceability, review, audit, and post-operation analysis.

+ Communicate active rules, temporary configurations, unresolved issues, and flights requiring continued monitoring during shift handover to ensure continuity of safe operations.

##### Specific procedure:

| Step | Task | Time | Notes |
| --- | --- | --- | --- |
| Step 1 Shift handover | Perform shift handover | 15 minutes before the end of preivous shift |  |
| Receive handover | Perform shift handover with the previous shift and other relevant positions. + Receive handover about the status of current situation from previous shift; + Receive handover regarding notes about the status of the systems (If the system is being upgraded/has downtime, the recipient must record the status and expected processing time); + Receive information about the list of airports which is setted with limitation and important information need to be followed. |  |  |
| Receive rules/ settings in use | Perform shift handover with the previous shift about the status of settings in automatically calculated rules in effect; The handover person presents the automatically calculated errors that arise during the shift to warn the next shift. |  |  |
| Step 2 Overall review | Perform review automatically calculated rules |  |  |
| Update information | Determine shift work plans: receive information about WX, NOTAM/ flight operations, update alternates, suitable and optimum routes to update in automatically calculated rules |  |  |
| Setting rules | Coordinate with the two above specific positions to set rules about traffic status, airport time slots cannot be selected as alternate, military activity status at airports and military areas, other information regarding rules setup. |  |  |
| Perform setup and activate rules | Perform setup, activate actions, rules and necessary information based on updated information above; Activate start all rules and settings after completing the setup of the shift. |  |  |
| Step 3 Flight preparation | Prepare documents for assigned flights | During the shift |  |
| Check relevant information | This step is referred to 3.1. End-to-end process flow |  | All documents must be prepared 03 hours before departure for DOM flights, 04 hours for INT flight |
| Step 4 Dispatch release | Produce flight plan and release |  |  |
|  | Use flight planning system to produce flight plan |  |  |
|  | Perform flight release refer to 3.1. End-to-end process flow |  |  |
| Step 5 Post-release change management | Monitor any information changes that impact to flight plan | Before flight departure |  |
|  | This step is referred to 3.1. End-to-end process flow |  |  |
| Step 6 Flight watch and flight following | Perform flight watch and flight following | After departure |  |
| Flight watch | This step is referred to 3.1. End-to-end process flow |  |  |
| Flight following | This step is referred to 3.1. End-to-end process flow |  |  |
| Step 7 In-flight information support | Support to provide necessary information to flight crew |  |  |
|  | This step is referred to 3.1. End-to-end process flow |  |  |
| Step 8 Coordination | Coordinate with positions specific in supervising and monitoring and specific in mastering NOTAM/ WX to update setting | During the shift |  |
|  | Coordinate with other positions, update assessment information every 3 hours to make necessary adjustments to suit the shift. |  |  |
|  | Track and handle error flights after running automatically, record and report to the assigned group if the error status is beyond handling capacity (for example: flight has been sent but not shown on EFB,...). |  |  |
|  | During shift, assist other positions with ad-hoc errors during OFP preparation. |  |  |
| Step 9 Detect, assess, and manage rules | Perform management rules | During the shift |  |
|  | Upon identification of an operational situation, this position shall: - Assess the potential impact on: + Flight safety and legality; - Take action to adjust rules and settings: Direct or correct actions within assigned authority to mitigate risks and maintain operational continuity. |  |  |
| Step 10 Documentation and shift continuity | Prepare for next shift handover | End of shift |  |
|  | Ensure that significant operational developments, decisions, and actions are: + Properly recorded in the shift handover file or log. |  |  |

##### Qualifications:

This position requires all staff must be at least 21 years of age. Minimum education requirements is University education. Good verbal and written in English. Undergo the fundamental aviation course by aviation institutes from Vietnam or international institutes or FTC, and other training courses as required by the Training Manual of VNA. Hold a Flight dispatcher license by the CAAV.

#### 3.2.4. Flight dispatcher – Specific role in controlling automatically calculated flights

##### Description of main roles:

This position monitors flights processed through Vietnam Airlines–approved automatic operational systems to ensure outputs are correctly generated and operationally valid in compliance with the AOC and applicable CAAV/company regulations. The position identifies any anomalies, failures, or inconsistencies in automated processing, promptly reports such issues to the appropriate supervisory or higher-level position, and performs necessary manual adjustments for the affected individual flight to ensure continued safety, legality, and operational compliance. This position is designated as a Flight Dispatcher (FD).

##### Responsibilities:

The unit is manned 24 hours per day and 7 days per week.

- Procedural compliance and professional judgment

+ Perform duties in accordance with approved procedures governing the use of automated systems.

+ Exercise professional judgment when system outputs appear unsuitable, inconsistent, or non-representative of actual operational conditions.

- Automatic flight monitoring

+ Monitor flights processed through approved automated flight planning and operational systems to verify that outputs are correctly generated, operationally valid, and suitable for use.

+ Ensure automated outputs remain compliant with the AOC, applicable CAAV regulations, and company-approved procedures.

- Report and escalation

+ Promptly report system anomalies, automation failures, or operational inconsistencies to the appropriate supervisory or higher-level position.

+ Escalate issues that may affect flight safety, legality, or operational continuity in accordance with established reporting lines.

- Operational awareness and information accuracy

+ Ensure that information used to validate automatic outputs is current, accurate, and consistent across systems.

- Coordination and information sharing

+ Coordinate with other positions, and other relevant operational functions to resolve automation-related issues.

+ Provide accurate and timely operational information to relevant stakeholders when automated outputs require clarification or adjustment.

- Shift continuity

+ Ensure that all manual adjustments, deviations from automated outputs, and identified system issues are properly recorded to support traceability, review, and post-flight analysis.

+ Communicate unresolved automation-related issues or flights requiring continued monitoring during shift handover to ensure continuity of safe operations.

##### Specific procedure:

| Step | Task | Time | Notes |
| --- | --- | --- | --- |
| Step 1 Shift handover | Perform shift handover | 15 minutes before the end of preivous shift |  |
| Receive handover | Perform shift handover with the previous shift and other relevant positions. Receive information on weather, military activity in the area, NOTAMs, extra fuel, MEL/CDL, and other information. |  |  |
| Step 2 Overall review | Perform overall review for all domestic flights |  |  |
|  | Crosscheck with other positions about the important information which need to be noted |  |  |
| Step 3 Flight preparation and dispatch release | Prepare documents for assigned flights | During the shift |  |
| Check calculated OFP | Check automatically calculted OFP in accordance with received information and ensure that all OFPs are uploaded on EFB |  | All documents must be prepared 03 hours before departure for DOM flights, 04 hours for INT flight |
|  | Take needed action to ensure all automatically calculted OFP accurate and follow all information and regulations. |  |  |
|  | Perform flight release refer to 3.1. End-to-end process flow |  |  |
| Step 4 Post-release change management | Monitor any information changes that impact to flight plan | Before flight departure |  |
|  | This step is referred to 3.1. End-to-end process flow |  |  |
| Step 5 Follow calculated flights | Follow all automatically calculated flights | During the shift |  |
|  | Perform follow and ensure that all automatically calculated flights with full of documents, including NOTAM, WX and DSP briefing |  |  |
|  | Request support from position specific in controlling rules if any case related to calculated flights has been raised. |  |  |
|  | Notify relevant IT support about the case there is no OFP on EFB although it is calculated and uploaded. |  |  |
| Step 6 Documentation and shift continuity | Prepare for next shift handover | End of shift |  |
|  | Ensure that significant operational developments, decisions, and actions are: + Properly recorded in the shift handover file or log. |  |  |

##### Qualifications:

This position requires all staff must be at least 21 years of age. Minimum education requirements is University education. Good verbal and written in English. Undergo the fundamental aviation course by aviation institutes from Vietnam or international institutes or FTC, and other training courses as required by the Training Manual of VNA. Hold a Flight dispatcher license by the CAAV.

### 3.3. VIP flight procedure

#### 3.3.1. Applicability:

The flight dispatchers, who are responsible for the VIP flight, will be approved annually by the CAAV;

The flight dispatchers are assigned specific duties.

#### 3.3.2. Flight route assignment

##### Flight dispatchers – specific role in supervising and monitoring:

This position takes responsibility for these flights:

+ Long-haul flights with a flight time of 8 hours or more;

+ Flights assessed as operationally complex, including operations to airports with limited communication capabilities;

+ Flights without an accompanying Dispatcher (DSP);

+ Flights specifically assigned to the Supervisor.

##### Other positions:

Other positions can take responsibility for these flights:

+ Flights with a flight time of less than 8 hours;

+ Flights assigned in accordance with the duty roster.

#### 3.3.3. Preparation process

| Step | Task | Time | Notes |
| --- | --- | --- | --- |
| Step 1 Assignment | Check and verify personal information | Receiving assignment to serve on a flight |  |
|  | Verify personal passport validity, including expiry date, availability of blank pages, and physical condition (e.g. damage, excessive wear, stains); |  |  |
|  | Ensure that the flight dispatcher license and relevant certificates remain valid; |  |  |
|  | Provide personal information to the supervisor when required; |  |  |
|  | Support the FOE division in route development, including careful note-taking and preparation of flight-related reference materials. |  |  |
| Step 2 Official confirmation of assignment | Post-assignment flight preparation | Receiving official confirmation of assignment |  |
|  | Submit personal documents for visa application, passport processing, or other required travel documentation as instructed; |  |  |
|  | Proactively review and familiarize himself/herself with flight-related information from official and reliable sources; |  |  |
| Step 3 Coordinate and gather information | Actively contact and gather information |  |  |
| FOE | Detailed plan: flight route, flight time, aircraft, flight path, expected load, conditions of arrival/alternative/ENR ALT airports. |  |  |
| Flight schedule and assignment team (OCD) | Official aircraft, reserve aircraft. Potential impact on other flights when an aircraft is "withdrawn" for VIP/charter flights. |  |  |
| Flight permit team (OCD) | Flight permit/slot for the arrival airport, overflight permit based on route FOE established. |  |  |
| Office of Corporation | Flight crew composition, VNA delegation, expected number of passengers. |  |  |
| Representative at the arrival airport (if any); | Expected parking position. Procedures and service capacity at the arrival airport (services, security, reception, etc.). |  |  |
| CLC | Accurate load and passenger information at each point in time. |  |  |
| Step 4 Follow and prepare | Continue follow any updates from units and prepare slide for presentation |  |  |
|  | Support the FOE in detailed route planning activities, including: + Thorough review of proposed routes in accordance with AIP and NOTAM, with particular attention to planned flight hours (FH) and block hours (BH); + Verification of all countries, territories, and territorial waters along the intended route of flight; + Validation of route data in the database to ensure no RDB TIME errors; any discrepancies shall be coordinated with the FOE for resolution; |  |  |
|  | In case of changes to flight schedule, re-verify the entire routing, departure and arrival slots, overflight and landing permits, and other related requirements; |  |  |
|  | Maintain clear and detailed records of all preparatory work performed; |  |  |
|  | Prepare presentation slides summarizing preparation activities for VIP flights. |  |  |

#### 3.3.4. Guidance on preparing VIP briefing slide

Upon approval of the official routing plan, the assigned flight dispatcher shall commence preparation of the VIP briefing slides using the latest available information at the time of drafting.

The assigned flight dispatcher shall periodically review and update all relevant briefing information and incorporate updates into the slides.

When the VIP preparation briefing schedule for company level is issued, the assigned flight dispatcher shall finalize the presentation (updated to the time of submission) according to the following milestones:

+ 72 hours (or 48 hours) prior to the company briefing date: submit the draft slides to the management of department for review and approval; the assigned flight dispatcher shall present the content and revise the slides in accordance with managerial feedback.

+ 48 hours (or 24 hours) prior to the company briefing date: finalize the slides (after department-level revisions) and submit them to the board of directors of OCC.

On the VIP preparation briefing date, the assigned flight dispatcher shall accompany OCC leadership to provide briefing support as required.

NOTE:

+ Use a standardized slide template, including consistent layout, typography (font, font size), and background.

+ Apply consistent terminology and wording conventions across all slides.

+ Clearly indicate the date and time of the latest information update.

+ Highlight key information (e.g., bold/mark critical items) where emphasis is required.

+ Clearly distinguish local time and UTC.

+ Exercise extreme caution when recording and presenting departure/arrival times, especially for different time zones or flights crossing midnight/date boundaries.

+ Carefully verify all countries, territories, and areas along the route, including any overlapping territorial waters or disputed jurisdictions.

#### 3.3.5. Day of operation

| Step | Description | Time | Notes |
| --- | --- | --- | --- |
| The designated flight dispatcher accompanying the aircraft |  |  |  |
| 1 | Produce and update OFP based on the latest ETD |  |  |
| 2 | Prior to filing the ATC FPL, confirm in the ATC database that FOE has changed status from IS to IN and has inserted STS/HEAD. If not yet completed, request FOE to update the database immediately. |  |  |
| 3 | File ATC FPL early. If required permits are not available within 24 hours before ETD, then filing ATC FPL immediately once all permits are obtained | 48 hours to 24 hours prior ETD |  |
| 4 | After filing, verify the ATC FPL status through available channels (e.g., AFTN, AMHS, SITA, email, or support from flight dispatcher at headquarter,…) |  |  |
| 5 | Update NOTAM and WX information, prepare the briefing package, and provide it to the flight crew. |  | For international flights, attach the document “COMMUNICATION PROCEDURE FOR VIP’S FLIGHT, BETWEEN CREW AND OCC” to the flight documentation package. |
| 6 | Respond to inquiries from the flight crew and the VIP delegation as required |  |  |
| 7 | Prepare and share the briefing package with the flight dispatcher team at OCD |  |  |
| 8 | Coordinate with the flight dispatcher at OCD to address any issues arising prior to departure |  |  |
| 9 | Remind the flight crew to conduct a communication test with OCC (SITA messages and/or SATCOM voice) |  |  |
| The duty flight dispatcher at OCD |  |  |  |
| Operational support and pre-departure preparation | The duty flight dispatcher – specific role in supervising and monitoring is responsible for supporting the accompanying flight dispatcher. |  |  |
|  | Monitor the ATC FPL status via AFTN, AMHS, email, etc. |  |  |
|  | Update NOTAM/ WX information when changes occur, and update/ send a revised OFP upon request. |  |  |
|  | Review thoroughly the briefing information prepared by the accompanying flight dispatcher and shared with the duty dispatch team. |  |  |
|  | For international flights, coordinate and support the communication test with the flight crew |  |  |
|  | The duty flight dispatcher – specific role in supervising and monitoring, together with DM, is responsible for the operational control and oversight of the VIP flight. |  |  |
| Handling contingencies/ operational issues | In the event of an issue, the flight dispatcher – specific role in supervising and monitoring (with support from other duty dispatchers) shall take the following actions: + Verify the information and its source(s); + Report to DM and duty leaders; + Assess the situation, consult relevant references, and/or contact the responsible authority directly to obtain full clarification; + Develop possible mitigation/solution options; + Select the most appropriate option; + Implement the selected option(s) until the issue is fully resolved |  |  |
| In-flight operational support and monitoring | The flight dispatchers, being specific role at supervising and monitoring and specific role at mastering NOTAM and WX, are responsible for flight tracking and in-flight monitoring of the flight |  |  |
|  | Ensure that all flight tracking systems are fully operational throughout the flight. |  |  |
|  | Ensure that all communication systems with the flight crew are continuously available, including verification of SITA alert audio levels. |  |  |
|  | Identify and monitor key operational milestones requiring support or updates, such as EDTO entry time, TAF updates, ETA revisions, and other critical phases of flight |  |  |
| In-flight information update and coordination | The flight dispatcher - specific role at mastering NOTAM and WX position is responsible for updating operational information, with flight dispatcher, specific role at supervising and monitoring, oversight and support. |  |  |
|  | Perform periodic checks and updates of: + NOTAM, including any newly issued or amended items; + Destination WX (TAF/METAR) upon issuance of new forecasts; + En-route weather affecting the planned route. |  |  |
|  | Provide the flight crew with the latest airport NOTAM (if any) and WX information for both the destination and landing alternate aerodromes. | At least 90 minutes prior to landing |  |
| Abnormal situations and contingency handling | Upon detection of any abnormal condition, establish communication with the flight crew via message or voice communication to verify the situation. |  |  |
|  | Once an abnormal situation affecting the flight is confirmed: + Report the situation to DM and Department Management; + Maintain continuous communication with the flight crew and provide operational support as required; + Execute instructions and guidance issued by management and supervisory authorities. |  |  |

#### 3.3.6. Post-Flight Actions and Reporting

The assigned flight dispatcher shall prepare and submit a post-flight report to the board of directors of OCC, and shall also complete the VIP flight report for submission to the Corporate Office (Head Office).

Coordinate with the FOE to update the database accordingly, including:

+ Changing the flight status from IN back to IS; and

+ Removing the STS/HEAD indicator, prior to releasing the aircraft back into regular commercial operations.

### 3.4. Procedure for flights over the uncontrolled airspace area

This procedure is applied for flights over the uncontrolled airspace area (Afganishtan)

| Step | Task | Time | Notes |
| --- | --- | --- | --- |
| Step 1 Flight preparation | Prepare as daily basis flights but focus on some following notice | Before departure |  |
| Select route | In addition to the standard preparation steps applied to daily flights, the flight dispatcher shall: + Select flights with higher operational efficiency, prioritizing options with better economic performance. |  |  |
| Apply BOBCAT | Apply for BOBCAT slot allocation. Normally, BOBCAT will allocate a slot on time as requested. |  |  |
| If BOBCAT not allocate as requested | There are 2 cases: + Case 1: At a later stage, reattempt to obtain a slot or access the system to identify an alternative slot with available flight levels; + Case 2: if no suitable slot is available, the flight shall be delayed in accordance with the assigned slot time, and the DM shall be informed. |  |  |
| Step 2 In-flight control and support | Assess and update information after departure | After departure |  |
| Traffic assessment and en-route risk evaluation | Continue to monitor VNA flights and regularly assess traffic conditions in the Pakistan and Turkmenistan regions to proactively identify traffic operating on routes similar to VNA flights. | After all flights have departed |  |
|  | Analyze and predict the potential impact of such traffic on VNA operations Criteria for identifying potentially conflicting traffic. Traffic shall be considered potentially impactful if it meets one or more of the following conditions: + Aircraft operating in the same direction and within 150NM or 20 minutes of flight time from the VNA aircraft; + Aircraft operating at the same level or within one adjacent flight level; + Obtain the callsign and verify, via Flightradar24, whether the aircraft has flown the same route through KABUL FIR during its last three flights. If the route is similar, such traffic shall be closely monitored. |  |  |
| Crew notification | When traffic posing a potential risk is identified, immediately inform the flight crew with the following details: + Callsign; + Current flight level; + Estimated distance from the VNA aircraft. |  |  |
| Step 3 En-route update prior to critical waypoints | Update important information for flight | Before entering critical waypoints |  |
|  | The flight dispatcher shall provide the flight crew with: + En-route weather assessment; + Updated FIR NOTAM; + Traffic situation within the KABUL FIR. | 30 minutes prior to reaching ULVAL/ZAMIR |  |
| Step 4 Flight watch and flight following | Perform flight watch and flight following |  |  |
| Flight watch | This step is referred to 3.1. End-to-end process flow |  |  |
| Flight following | This step is referred to 3.1. End-to-end process flow |  |  |

### 3.5. Aircraft delivery procedure

| Step | Task | Time | Notes |
| --- | --- | --- | --- |
| Step 1 Preparation | Perform preparation after the notification from HR Dept |  |  |
|  | OCD’s management assign FD who takes responsibility to conduct delivery flight | After the notification from HR Dept | Division management |
|  | Receiving Decision on assining personel abroad for business |  | Delivery member |
|  | Contacting Head of the delegation to receive information and get ready to submit VISA |  |  |
| Step 2 Data collection | Collect all necessary information for delivery flights |  |  |
|  | Contacting team leader to exchange necessary information to prepare for the ferry flight about sector, departure/ arrival airport, take-off/landing time, and techstop airport if any |  |  |
|  | Contacting member from SQD to get information about 24-bit |  |  |
|  | Contacting Head of Supervisory team for assisstance in obtaining information about flt ops personel of the partner (Boe, Airbus,..) |  |  |
|  | Sending information about the number of the delegation team and requesting partner to send draft weigh manifest |  |  |
|  | Calculating estimated DOW |  |  |
|  | Consulting with the division management and flight permission team about the flight number of ferry flight |  |  |
|  | Providing the following information to OPS team (FOE division) to develop a flight plan: + Aircarft type; + MSN; + Time, place of delivery; + Flight number; + Sector; + Departure airport/ Arrival airport; + ETD/ETA in UTC/LT; + Estimated PLD, DOW; + 24-bit information; + Techstop airport (if any); |  |  |
| Step 3 Develop a flight plan and deploy for related units | Develop a flight plan and deploy it for all related units |  |  |
|  | Develop a plan for receiving aircraft: develop airport and route analysis |  |  |
|  | Asking for opinions from division management and delegation team on the flight plan (if there is more than 01 flight plan) |  |  |
|  | Discussing and getting consultants on the ground handling of the Passenger Services Department regarding the techstop airport of the all flight plans |  |  |
|  | Preparing a proposal based on the flight plan from FOE and submit for signature |  |  |
|  | Getting approval and deploying for related units, includes: + Aircraft Assignment & Scheduling (OCD); + Flight Permission (OCD); + EFB Group (FOE); + Aircarft performance Group (FOE); |  | Consider to print QRH from Vietnam and bring it with you |
|  | Deploying to VIAGS to arrange personnel to receive handover documents and support making loadsheets according to VNA's template |  |  |
| Step 4 Work at delivery center | All needed works before and after ToT at delivery center |  |  |
| Before ToT | Coordinating with Delivery Team Leader to have enough information of the ToT time and ferry flight |  |  |
|  | Providing the the departure time and necessary information of ferry flight to flight dispatcher of partner |  |  |
|  | Requesting partner for assisstance in consulting about potable water, file ATC, print 02 set of flight documents (OFP, NOTAM, WX), and briefing with flight crew. |  |  |
|  | Providing eAPIS information to flight dispatcher of partner for the flight from USA. |  | Applied only for flight from USA |
|  | Coordinating with CAAV to check operation manual (RTOW, chart, enroute, SOP) |  |  |
|  | Coordinating with EFB Group to receiving checklist of part number. |  |  |
|  | Coordinating with member of Tech Dept. to receive Weigh and Balance, Chapter 2 and send to PER Group. |  |  |
|  | Updating and reporting to division management and related units about there are any changes in ToT time and ferry flight |  |  |
| After ToT | Receiving documents from partner |  |  |
|  | Coordinating with SQD to receive and check all certificates from CAAV |  |  |
|  | Scanning and sending all documents and copies of certificates to Flight permission Group and related units, includes EFB and OPS Group |  |  |
|  | Coordinating with VAECO and partner to check EFB, includes LIDO eRM, OPT/Flysmart và Nav |  |  |
|  | Informing EFB Group after checking and updating the necessary information if manuals/data are out of date |  |  |
|  | Receiving flight permission |  |  |
|  | Preparing flight permission document and send to related units, includes: DM, VNA’s flight dispatcher, flight crew |  |  |
|  | Providing DOW, PLD to DSP at headquarter to produce OFP (if necessary); |  |  |
|  | Informing with DM about the departure/arrival time of ferry flight |  |  |
|  | Coordinating with SUP DSP to check and update flight documents before departure time |  |  |
|  | Briefing with flight crew with flight dispatcher of partner |  |  |
|  | Providing Final Weigh Manifest to CLC to make Loadsheet |  |  |
| Step 5 After the flight | Send document to related units |  |  |
|  | Sending documents, certificates to VIAGS |  |  |
|  | Sending operation documents to FOE |  |  |